PHYSICAL LAYOUT REPORT

NTT ASIC Improvement History

How we reduced antenna violations from 34 to 7, then to 1

Design: ntt_core_256_sram

Flow: OpenLane / OpenROAD with sky130A

Branch: Physical_Layout

Period: July 2026

Scope. The 34 -> 7 -> 1 numbers are net antenna violations. They are not a count of every signoff issue in the design.

# Progress at a glance

| Run date | Net violations | Pin violations | What this run showed |
| --- | --- | --- | --- |
| July 8 | 34 | 35 | The broad conservative rerun did not solve antenna. |
| July 19 | 7 | 7 | Stronger automatic diode repair produced the large improvement. |
| July 29 | 1 | 1 | Three small, report-driven ECO rounds produced the best result. |

Source: local OpenLane metrics and OpenROAD antenna reports from the named runs.

# 1. Starting point: 34 violations

The July 8 run, RUN_2026.07.08_asic_conversion_pulled_openlane, finished with 35 violating pins on 34 nets. We had lowered core utilization to 20% and placement density to 0.35 to create more whitespace. That was a reasonable first test, but the routed result showed that whitespace alone was not enough. Antenna depends on the exact route and where protection diodes connect.

# 2. The large improvement: 34 to 7

For RUN_2026.07.19_physical_antenna_only_v2, we returned to the stronger Physical_Layout baseline and focused the flow on antenna repair. We used diode insertion strategy 3, a 90 antenna margin, 10 global routing diode iterations, heuristic diode insertion, and a 250 heuristic threshold. The run reported 9,418 diode cells instead of 1,963 in the July 8 run. This reduced the result to 7 violating pins on 7 nets, with zero detailed-route DRC violations.

## Main OpenLane settings

| Setting | Value | Why we used it |
| --- | --- | --- |
| DIODE_INSERTION_STRATEGY | 3 | Use stronger automatic antenna-diode insertion. |
| GRT_ANT_MARGIN | 90 | Begin repair before a route reaches the antenna limit. |
| GRT_MAX_DIODE_INS_ITERS | 10 | Give global routing more chances to repair nets. |
| RUN_HEURISTIC_DIODE_INSERTION HEURISTIC_ANTENNA_THRESHOLD | 1 250 | Insert protection proactively on nets likely to violate. |
| FP_CORE_UTIL PL_TARGET_DENSITY | 25 0.45 | Return to the proven placement baseline after 20 / 0.35 made antenna worse. |

Supporting options kept enabled: SYNTH_BUFFERING = 1 and both global- and placement-resizer timing optimizations = 1.

What we learned. The automatic settings produced the 34 -> 7 improvement. From 7 onward, the targeted ECO diode placement produced the better result.

# 3. Targeted cleanup: 7 to 1

We read the antenna report after each run, placed small groups of antenna diodes close to the violating sink pins, rerouted, and checked both antenna and detailed DRC. This kept each experiment small enough to understand.

ECO1: 7 -> 5: Six diodes were placed near the u[24] and u[26] sink areas. After rerouting, five nets remained and detailed-route DRC stayed at zero.

ECO2: 5 -> 2: Fifteen diodes were added around the five remaining violating nets. The next antenna report contained only _0976_ and net314. Detailed-route DRC stayed at zero.

ECO3: 2 -> 1: Six more diodes targeted _0976_ and net314. Both were cleared. Rerouting moved the last violation back to butterfly_inst.u[26], leaving one pin on one net.

# Why we kept ECO3

We also tried follow-up experiments around the last u[26] problem, including a buffer-based change and higher-metal routing guides. Some experiments moved the problem or created new violations, so they were not accepted as improvements. ECO3 remained the best measured balance: one antenna violation and zero OpenROAD detailed-route DRC violations.

# Current best result

| Best source run | RUN_2026.07.29_physical_v2_targeted_eco3 |
| --- | --- |
| OpenROAD antenna | 1 pin / 1 net |
| Remaining net and pin | butterfly_inst.u[26] at _5459_/A1 on met1 |
| Detailed-route DRC | 0 violations |
| Timing signoff | Setup WNS/TNS 0.00 ns / 0.00 ns |
| Separate signoff work | 4 Magic met3 minimum-area markers; 2 LVS errors |

The separate Magic DRC and LVS findings mean the design is improved, but not yet tapeout-clean.

# Main lesson

The biggest improvement came from stronger automatic diode insertion. The final improvement came from reading the actual violator list and fixing a few sink areas at a time. The next goal is to clear the last u[26] antenna violation without disturbing the clean detailed routing, then finish the remaining Magic DRC and LVS work.